SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2- PSEUDOCODE WRITING TASK (ALGORITHM DESIGN)

| Course Code: | CSA1208 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL2- PSEUDOCODE WRITING TASK (ALGORITHM DESIGN) |
| Weightage: | 30% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | SURIYA SRI S | Reg.No.: | 1925211218 |
| Department: | B.E | Date: | 2:09:2026 |

ANNEXURE A

Pseudocode Writing Task (Algorithm Design)

1. Write pseudocode to design a DMA-based data transfer algorithm that moves data from an I/O device to memory while handling interrupts and minimizing CPU involvement.

2. Develop pseudocode for an interrupt handling system that prioritizes vectored interrupts from multiple devices and dispatches appropriate service routines.

3. Write pseudocode to select between memory-mapped I/O and I/O-mapped I/O dynamically based on latency and bandwidth requirements.

4. Design pseudocode for a bus arbitration algorithm that manages multiple devices communicating over PCIe, USB, and Thunderbolt interfaces.

5. Write pseudocode to implement a hybrid I/O communication mechanism that uses polling for low-speed devices and DMA with interrupts for high-speed devices.

Code of Conduct:

I  SURIYA SRI S(192511218) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

SURIYA SRI S

ANSWERS – PSEUDOCODE WRITING TASK

QUESTION 1

DMA-Based Data Transfer from I/O Device to Memory

Problem Understanding

The algorithm must move a block of data from an I/O device to a memory buffer with minimum CPU involvement. The CPU should configure the DMA controller, allow the transfer to proceed independently, and respond only when the DMA controller generates a completion or error interrupt. This reduces CPU overhead and is suitable for continuous or high-volume I/O.

Input: I/O device address, destination memory buffer, transfer size, transfer mode, and interrupt configuration.

Output: data stored correctly in memory, along with completion or error status.

Main control structures: initialization, validation, conditional checks, and an interrupt service routine.

Algorithm Design

The DMA controller is configured with the source device, destination memory address, transfer count, direction, and interrupt enable bit. After the transfer starts, the CPU is free to execute other tasks. When DMA completes or detects an error, it raises an interrupt. The ISR checks the status, clears the interrupt, updates the buffer state, and records the result.

Pseudocode

ALGORITHM DMA_IO_TO_MEMORY_TRANSFER

INPUT: device, memory_buffer, transfer_size

OUTPUT: transfer_status

INITIALIZE DMA_controller

VALIDATE device, memory_buffer, transfer_size

IF validation fails THEN

RETURN ERROR_INVALID_PARAMETER

END IF

CONFIGURE DMA.source      <- device.data_register

CONFIGURE DMA.destination <- memory_buffer

CONFIGURE DMA.count       <- transfer_size

CONFIGURE DMA.direction   <- IO_TO_MEMORY

CONFIGURE DMA.mode        <- BLOCK_TRANSFER

ENABLE DMA completion and error interrupts

CLEAR previous DMA status flags

SET transfer_status <- BUSY

START DMA

WHILE transfer_status = BUSY DO

CPU performs other useful tasks

END WHILE

INTERRUPT SERVICE ROUTINE DMA_ISR

READ DMA.status

IF DMA.error_flag = TRUE THEN

CLEAR DMA error flag

SET transfer_status <- ERROR

SIGNAL transfer failure

ELSE IF DMA.complete_flag = TRUE THEN

CLEAR DMA complete flag

VALIDATE received data/buffer status

SET transfer_status <- COMPLETE

SIGNAL transfer completion

END IF

RETURN FROM INTERRUPT

END ISR

RETURN transfer_status

END ALGORITHM

Working Explanation

The CPU initializes the DMA controller only once for the current transfer.

The DMA controller becomes responsible for moving the data between the I/O device and memory.

The CPU does not poll every byte or word, so processor utilization is reduced.

A completion interrupt informs the CPU that the transfer has finished.

An error interrupt allows the system to recover without corrupting the buffer silently.

Why the Algorithm Minimizes CPU Involvement

The CPU performs configuration, not bulk copying. The DMA hardware handles the actual data movement, while a single interrupt can represent completion of a large transfer. The approach therefore scales better than byte-by-byte interrupt-driven I/O.

QUESTION 2

Prioritized Vectored Interrupt Handling for Multiple Devices

Problem Understanding

Multiple devices can request service at the same time. The interrupt system must identify the requesting device, prioritize vectored interrupt requests, and directly dispatch the corresponding service routine. Vectored interrupts reduce the need for software to search all devices after every interrupt.

Input: pending interrupt requests, vector number, device priority, and interrupt vector table.

Output: correct service routine executed for each accepted interrupt.

High-priority requests are serviced before lower-priority requests when multiple requests are pending.

Algorithm Design

A programmable interrupt controller or equivalent hardware maintains pending requests and priorities. The CPU obtains the interrupt vector of the highest-priority pending device. The vector is used as an index into the service routine table. After service, the request is acknowledged and the next pending request is handled.

Pseudocode

ALGORITHM VECTORED_INTERRUPT_DISPATCH

INPUT: device_list, vector_table, priority_table

OUTPUT: interrupt_service_result

INITIALIZE interrupt_controller

FOR each device IN device_list DO

REGISTER device.vector

REGISTER device.priority

ENABLE device interrupt

END FOR

LOOP FOREVER

IF interrupt_controller.has_pending_request() = FALSE THEN

CONTINUE

END IF

pending_set <- interrupt_controller.get_pending_requests()

selected <- SELECT request IN pending_set WITH HIGHEST priority

vector <- selected.vector

SAVE minimal CPU context

ACKNOWLEDGE selected request

service_routine <- vector_table[vector]

IF service_routine EXISTS THEN

CALL service_routine(selected.device)

SET interrupt_service_result <- SUCCESS

ELSE

LOG unknown vector condition

SET interrupt_service_result <- ERROR_UNKNOWN_VECTOR

END IF

RESTORE CPU context

RETURN FROM INTERRUPT

END LOOP

END ALGORITHM

Priority and Dispatch Rules

Each interrupt source has a unique vector or vector index.

Priority is evaluated before dispatch when several requests are pending.

The service routine should perform only the urgent work required for the device.

Long processing can be deferred to a background task so that higher-priority interrupts are not blocked.

Example

Assume three devices: a high-speed network interface with priority 1, a storage controller with priority 2, and a keyboard with priority 3. If all three interrupt requests arrive together, the vector for the network device is selected first, followed by storage and then keyboard. This provides deterministic priority-based handling.

QUESTION 3

Dynamic Selection Between Memory-Mapped I/O and I/O-Mapped I/O

Problem Understanding

The system must select an I/O addressing technique according to application requirements. Memory-mapped I/O places device registers in the processor address space and can use normal load/store instructions. I/O-mapped I/O uses a separate I/O address space and dedicated I/O instructions on architectures that support it. The selection algorithm compares required latency and bandwidth against configured thresholds.

Input: required latency Lreq, required bandwidth Breq, available mapping modes, latency estimates, and bandwidth estimates.

Output: selected mode - MEMORY_MAPPED_IO or IO_MAPPED_IO.

Decision principle: choose the mode that satisfies both requirements; if both satisfy them, choose the one with lower weighted performance cost.

Decision Algorithm

A threshold-based method is simple and explainable. Memory-mapped I/O is preferred for very low latency or high-bandwidth register access when it provides the required performance. I/O-mapped I/O is selected when its overhead is acceptable and a separate I/O address space is preferred.

Pseudocode

ALGORITHM SELECT_IO_MAPPING

INPUT: Lreq, Breq

MMIO_latency, MMIO_bandwidth

PMIO_latency, PMIO_bandwidth

latency_weight, bandwidth_weight

OUTPUT: selected_mode

FUNCTION satisfies(latency, bandwidth)

RETURN (latency <= Lreq) AND (bandwidth >= Breq)

END FUNCTION

mmio_ok <- satisfies(MMIO_latency, MMIO_bandwidth)

pmio_ok <- satisfies(PMIO_latency, PMIO_bandwidth)

IF mmio_ok = TRUE AND pmio_ok = FALSE THEN

selected_mode <- MEMORY_MAPPED_IO

ELSE IF pmio_ok = TRUE AND mmio_ok = FALSE THEN

selected_mode <- IO_MAPPED_IO

ELSE IF mmio_ok = TRUE AND pmio_ok = TRUE THEN

mmio_cost <- latency_weight * MMIO_latency

+ bandwidth_weight * (1 / MMIO_bandwidth)

pmio_cost <- latency_weight * PMIO_latency

+ bandwidth_weight * (1 / PMIO_bandwidth)

IF mmio_cost <= pmio_cost THEN

selected_mode <- MEMORY_MAPPED_IO

ELSE

selected_mode <- IO_MAPPED_IO

END IF

ELSE

selected_mode <- NO_VALID_MODE

END IF

RETURN selected_mode

END ALGORITHM

Dynamic Operation

Read the current application requirements instead of using a fixed I/O mode for every device.

Estimate or obtain the performance characteristics of both addressing methods.

Reject any mode that fails either latency or bandwidth requirements.

If both are acceptable, compare a weighted cost and select the lower-cost option.

Re-evaluate the choice when the workload changes significantly.

Example

For a control device requiring very low response time, the algorithm is likely to select memory-mapped I/O when its measured latency is lower. For a slower peripheral where latency and bandwidth requirements are modest and I/O-space access is sufficient, I/O-mapped I/O may be selected. The algorithm remains dynamic because the choice is driven by requirements and measured characteristics.

QUESTION 4

Bus Arbitration for PCIe, USB, and Thunderbolt Devices

Problem Understanding

The system may contain multiple devices with different traffic characteristics. The arbitration algorithm must decide which request receives service next while respecting interface-specific priorities and preventing one requester from monopolizing service. The design uses a common logical scheduler with queues for PCIe, USB, and Thunderbolt traffic.

PCIe traffic is organized through the host/root-complex and endpoint fabric; requests can be scheduled by priority and queue state.

USB transfers are scheduled by the host controller, with timing requirements for periodic traffic.

Thunderbolt devices can carry high-bandwidth traffic through a switched fabric and need fair bandwidth allocation.

The pseudocode models a system-level arbitration policy above the interface-specific controller behavior.

Algorithm Design

The arbiter maintains separate queues, assigns weights, gives urgent or time-sensitive requests higher priority, and uses round-robin or weighted round-robin among comparable requests. A service quantum limits the number of bytes or transactions granted in one turn, improving fairness.

Pseudocode

ALGORITHM MULTI_INTERFACE_BUS_ARBITRATION

INPUT: request_queues[PCIe, USB, THUNDERBOLT]

weight[PCIe, USB, THUNDERBOLT]

service_quantum

OUTPUT: grants

INITIALIZE deficit[PCIe] <- 0

INITIALIZE deficit[USB] <- 0

INITIALIZE deficit[THUNDERBOLT] <- 0

last_selected <- THUNDERBOLT

LOOP FOREVER

FOR each interface IN [PCIe, USB, THUNDERBOLT] DO

deficit[interface] <- deficit[interface] + weight[interface]

END FOR

urgent <- FIND time-critical pending request

IF urgent EXISTS THEN

selected <- urgent.interface

ELSE

selected <- NEXT non-empty interface after last_selected

with maximum eligible deficit

END IF

IF selected = USB AND periodic transfer deadline is near THEN

grant <- schedule required periodic USB transaction

ELSE

grant <- MIN(service_quantum, selected.pending_bytes)

END IF

SEND grant(selected, grant)

UPDATE selected queue and counters

deficit[selected] <- deficit[selected] - grant_cost

last_selected <- selected

IF selected queue becomes empty THEN

RESET selected deficit to 0 when required

END IF

END LOOP

END ALGORITHM

Fairness Mechanism

Weighted service gives high-throughput interfaces enough service without permanently blocking USB or other traffic.

Round-robin ordering among eligible queues prevents repeated selection of the same requester.

A bounded service quantum prevents a single device from consuming the complete service opportunity.

Urgent periodic transfers can be promoted when deadlines are close.

Result

The algorithm combines priority and fairness. It does not treat all interfaces as electrically identical; instead, it provides a scheduling policy that can sit above the interface controllers while allowing PCIe, USB, and Thunderbolt controllers to enforce their own protocol rules.

QUESTION 5

Hybrid I/O: Polling for Low-Speed Devices and DMA with Interrupts for High-Speed Devices

Problem Understanding

A hybrid design avoids wasting CPU time on high-speed data transfers while keeping the control logic for low-speed devices simple. Low-speed devices are checked periodically by polling because their event rate is small. High-speed devices use DMA for bulk movement and generate interrupts only for completion or error conditions.

Low-speed devices: keyboard, simple sensors, status ports, or low-rate control devices can be polled periodically.

High-speed devices: storage, network, data acquisition, or high-rate peripherals use DMA.

CPU involvement is concentrated in polling intervals and DMA interrupt service routines rather than byte-level transfers.

Algorithm Design

The controller maintains a device configuration table. Each device is classified by speed. Polling tasks are scheduled using a timer or periodic loop. For high-speed devices, DMA descriptors are configured and started. A DMA completion interrupt signals the CPU to process or hand over the completed buffer.

Pseudocode

ALGORITHM HYBRID_IO_MANAGER

INPUT: device_table, memory_buffers

OUTPUT: device_status

INITIALIZE timer

INITIALIZE DMA_controller

FOR each device IN device_table DO

IF device.speed = HIGH THEN

CONFIGURE DMA source <- device.data_register

CONFIGURE DMA destination <- memory_buffers[device]

CONFIGURE DMA count <- device.transfer_size

ENABLE DMA completion/error interrupt for device

START DMA for device

ELSE

CONFIGURE poll_interval for device

MARK device as POLL_MODE

END IF

END FOR

LOOP FOREVER

IF timer.poll_event() = TRUE THEN

FOR each device IN device_table WHERE device.speed = LOW DO

status <- READ device.status_register

IF status indicates new data THEN

READ available low-speed data

STORE data in device buffer

END IF

END FOR

END IF

// High-speed completion handled asynchronously

CONTINUE normal CPU work

END LOOP

INTERRUPT SERVICE ROUTINE HIGH_SPEED_DMA_ISR(device)

READ DMA.status(device)

IF DMA error THEN

CLEAR error

RECORD device_status <- ERROR

RESTART or REPORT according to recovery policy

ELSE IF DMA complete THEN

CLEAR completion flag

MARK buffer[device] <- READY

NOTIFY data-processing task

PREPARE next DMA descriptor

START next transfer if data is continuous

END IF

RETURN FROM INTERRUPT

END ISR

END ALGORITHM

Comparison of Polling and DMA with Interrupts

| Aspect | Polling for Low-Speed | DMA + Interrupts for High-Speed |
| --- | --- | --- |
| CPU work | Periodic status checking | Setup + completion/error ISR |
| Best use | Low event rate | Large or continuous data blocks |
| Data movement | CPU reads/writes data | DMA controller moves bulk data |
| Event handling | Timer-driven | Hardware interrupt-driven |
| Efficiency | Simple but wastes cycles if over-polled | Efficient for high data rates |

Why the Hybrid Approach Works

Polling is acceptable when devices change slowly because the CPU can check them at a low frequency without significant overhead. High-speed transfers would create excessive polling or interrupt frequency, so DMA is used for bulk movement and interrupts are reserved for important state changes such as transfer completion or errors. This combination balances responsiveness, CPU efficiency, and implementation simplicity.

INTEGRATED ALGORITHM ANALYSIS

The five algorithms cover the major I/O communication mechanisms addressed in the assessment. DMA is used when large data movement should occur without continuous CPU participation. Vectored interrupts provide fast and direct service routine dispatch. Dynamic I/O mapping selects an addressing technique according to application requirements. Bus arbitration provides fair service among different interface classes. The hybrid model combines polling and DMA according to device speed.

| Q | Main technique | Primary benefit | Key control structure |
| --- | --- | --- | --- |
| 1 | DMA transfer | Minimum CPU involvement | IF / WHILE / ISR |
| 2 | Vectored interrupts | Fast priority dispatch | FOR / IF / vector lookup |
| 3 | Dynamic I/O mapping | Requirement-based selection | IF / ELSE / function |
| 4 | Bus arbitration | Fair interface access | LOOP / FOR / priority / round-robin |
| 5 | Hybrid I/O | Balanced low/high-speed service | FOR / IF / timer / ISR |

Completeness and Error Handling

Invalid configuration values are rejected before a transfer or dispatch operation begins.

DMA completion and error interrupts are handled separately.

Unknown interrupt vectors are reported instead of silently ignored.

The dynamic I/O selector can return NO_VALID_MODE when neither mapping satisfies the requested requirements.

The bus arbiter uses bounded service quantities to reduce starvation.

The hybrid manager can restart or report a DMA fault according to the configured recovery policy.

Conclusion

The proposed pseudocode designs convert the architectural requirements into clear, executable-style algorithms. Each solution uses appropriate control structures and explicitly handles normal operation as well as common error or edge cases. The overall approach reduces unnecessary CPU work, improves responsiveness, and provides a structured way to manage multiple I/O interfaces and interrupt sources.

MARKS ALLOCATION

|  | Problem Understanding (20%) | Algorithm Design (30%) | Use of Control Structures (20%) | Pseudocode Structure (10%) | Completeness (20%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Identifies all inputs, outputs, constraints accurately | Minor missing elements | Partial understanding | Incorrect interpretation |
| Algorithm Design | 30% | Complete, correct, and logically sound solution | Minor logical gaps | Partially correct logic | Incorrect approach |
| Use of Control Structures | 20% | Efficient and appropriate use of loops, conditions, functions/modules | Minor inefficiencies | Limited or basic usage | Incorrect or missing constructs |
| Pseudocode Structure | 10% | Well-structured, clear, consistent format, easy to follow | Mostly clear with minor issues | Difficult to follow in parts | Poorly structured and unclear |
| Completeness | 20% | Handles all cases; optimized approach | Handles most cases; reasonable efficiency | Handles basic cases only | Incomplete or inefficient |